Scope

Develop and release an application for mixer data logging and display, equivalent to ZSD (Kassel) current PVS application.

- SCI will charge for single time development and release.

- Zeppelin (Kassel) will be the owner of the application and will distribute the application to their customers.

- Source codes will be the property of Zeppelin (Kassel). (Need to discuss internally if we will use some proprietary components in this)

- For future support/bug fixes/feature addition, Zeppelin (Kassel) will raise new service order separately.

Tech Stack (Decided after analysing all available options)

Front End* – React.js
Back End – .NET
Database – SQL Server

- UI should be on web application, so that it can be accessed remotely from network.

- Application should be compatible with recent Windows OS.

*For front end, standard NAMiQ UI/UX design is required to be followed. Ricardo Klement from ZSD will develop UI designs/wireframes.

System Requirements

- Application should be lightweight and should be able to run on,

  - Medium spec PCs/laptops.

  - Siemen’s Panel PCs.

Functional Scope (Phase 1)

- OPC UA Communication (OPC UA Client inbuilt at application level, OPC UA Server at PLC side)

- Configuration – Mixers, Parameters, PLC Tags.

- Data logging (online values of configured list of parameters to be logged in database cyclically). Minimum required frequency for logging is 1 seconds, but any higher frequency (0.5 seconds to 0.1 seconds) will be good to explore the performance option.

- Graph Display for parameter

  - Online (real-time) graph/trend display for parameters.

  - History graph/trend display for parameters.

  - Graph/Trend customization options (similar to general SCADA trends)

    - X-axis/Y-axis span

    - Parameter selection for trend channel

    - Channel color selection.

    - Zoom in/out

    - Cursor line

    - Legends

    - Other common options

  - History data selection on the basis of Mixer, Date/Time, Batch No./Id.

  - Comparison of history batches (superimposing of parameter graph for multiple batches)

- Process screen showing PLC tags’ real-time values

  - Real time parameters values in grid format.

  - Parameter selection by user

- User Management – Role based user management

- Licensing

  - Instance based perpetual licensing

  - Hardware binding with Machine Id.

Functional Scope (Phase 2) (detailed scope to be decided later)

- Recipe Management

- Production Order

- Production Order/Recipe transfer to PLC

- Batch report logging (at the end of each batch)

- Batch report display